✅ PROJECT PLAN — Dual-Camera Face ID System (Embedded + ML)

1. Introduction

This project builds a Face ID–like authentication system using an embedded board and a dual-camera setup:

- RGB camera → captures texture

- IR / Depth camera → captures depth map or structured light pattern

Goal:

- Perform face detection, 3D liveness verification, and face recognition directly on an embedded processor (e.g., Raspberry Pi / Jetson Nano / ESP32-CAM + depth sensor).

- Prevent spoofing (photos, videos) using depth + IR pattern analysis.

- Use a local ML pipeline for fast authentication.

This mimics the TrueDepth system used in modern devices, but in a simplified DIY embedded style.

2. Hardware Requirements

✔ 1. Main Processor (choose one)

- NVIDIA Jetson Nano / Orin Nano → best for ML inference

- Raspberry Pi 4 / Pi 5 → balanced choice

- ESP32 + External Depth Camera → lightweight version (limited ML)

✔ 2. Cameras

You need two synchronized inputs:

A. RGB Camera

- Raspberry Pi Camera Module v3

- USB webcam with 1080p/720p support

B. Depth / IR Camera

Choose one of the following:

- RealSense D435 / D415 (best, gives depth map)

- ESP32-S3 Eye IR Cam (cheap IR-only version)

- OV5647 + IR Illuminators

- Arducam Time-of-Flight (ToF) camera

If you want Face-ID-like accuracy, RealSense or Arducam ToF is recommended.

✔ 3. Additional Hardware

- IR flood illuminator (850nm)

- IMU (optional for device motion compensation)

- Cooling fan for Jetson

- SD card, power adapter

- Mount/3D printed frame to keep both cameras aligned

3. Software Requirements

✔ Embedded OS / Runtime

- JetPack (for Jetson)

- Raspberry Pi OS

- ESP-IDF (for ESP32 version)

✔ Libraries

- OpenCV

- Dlib or InsightFace

- PyTorch / ONNX Runtime

- Numpy

- Depth API (RealSense SDK or ToF driver)

4. Software Pipeline (Detailed Step-by-Step)

This is the full workflow your system will follow.

A. Capture Stage

- Synchronize RGB + depth frames

- Calibrate cameras (intrinsic + extrinsic)

- Remove lens distortion

- Align depth map to RGB frame

B. Preprocessing

- Convert image to normalized tensor

- Resize to model input size (112×112 or 160×160)

- Apply histogram correction

- Smooth depth map (bilateral filtering)

C. Face Detection

Use a lightweight detector:

- RetinaFace MobileNet

- BlazeFace

- YOLO-Nano

Output → bounding box + face landmarks.

D. 3D Liveness Detection

Use the depth image to verify the face is real and 3D, not a photo/video.

Methods you can implement:

1. Depth Histogram Method

- Real faces → continuous depth gradient

- Printed photos → flat depth plane

2. Depth Edge Variation

- Human face has nose peaks, eye sockets, curved structure

- Fake images don’t

3. ML Classifier (best)

- Use a small CNN that takes:

  - RGB crop

  - Depth crop

  - IR pattern response

Output → real / spoof.

E. Face Embeddings

Use a strong model (optimized to run on embedded):

- MobileFaceNet

- GhostFaceNet

- ArcFace ONNX (quantized)

Extract a 128D or 512D embedding vector.

F. Authentication

During registration:

- Store the user’s embedding in a secure local DB
During login:

- Compare cosine distance / Euclidean distance

- If distance < threshold → authenticated

- Depth+liveness must also pass

5. Dataset Plan

To train liveness/recognition:

✔ 1. RGB Face Dataset

Use:

- VGGFace2

- CelebA

- LFW

- CASIA-WebFace

✔ 2. Depth / IR Dataset

Use:

- CASIA-SURF (RGB+Depth+IR)

- MLFP (multi-modal face anti-spoofing)

- REAL-SENSE anti-spoofing datasets

✔ 3. Custom Dataset

You must record:

- 10–20 frontal RGB+Depth frames per user

- Spoof samples (printed photos, screen replay)

- Various lighting conditions

6. Machine Learning Methods Used

✔ Face Detection

- RetinaFace (light model)

✔ Face Recognition

- ArcFace / MobileFaceNet

- Embedding-based nearest-neighbor classification

✔ Liveness

Use multimodal ML model:

- 1D CNN for depth signals

- 2D CNN for IR image patterns

- Fusion network to combine RGB + depth + IR

7. Final Deliverables Checklist

Hardware

- Synchronized dual-camera rig

- IR+Depth calibration

- Working embedded board setup

Firmware / Driver Layer

- Depth SDK working

- Synchronized capture code

Software

- Face detection pipeline

- Liveness detection

- Face recognition

- Storage & authentication system

Dataset

- Custom + public dataset

- Preprocessed and aligned samples

ML

- ONNX-optimized model

- Quantized version for edge device

- Real-time inference (≥15 FPS)

Frontend (optional)

- Local display / web panel

- Registration + verification screen

- Logs and access history